Gold & Silver
Tag / Label Printing System

Architecture & Design Document

Mobile App  ·  ESP32 BLE Bridge  ·  TSC TTP-244 Pro
Prepared for: Jitendra
Version 0.1 · Draft

# 1. Executive Summary

This document describes a complete tag/label printing solution for the gold and silver industry. The solution combines a cross-platform mobile app (Flutter, Android + iOS), an ESP32-based wireless bridge, and the TSC TTP-244 Pro thermal label printer. The ESP32 simultaneously reads the live weight stream from any standard RS232 weighing scale and accepts print commands from the phone over Bluetooth Low Energy.

The mobile app gives the operator a drag-and-drop label designer with dynamic sizing, font selection, logos, QR codes, barcodes, prefix/unit configuration, and serial-number management — together with daily / date-range printing reports exportable to CSV or XLSX.

## Key capabilities

Dynamic label size (any W × H in millimetres) with on-screen WYSIWYG canvas.

Drag-and-drop placement of text, weight, serial, date/time, QR, barcode, box, and logo elements.

TSPL font selection (1–8) with X/Y scaling and 0/90/180/270 rotation.

Live Gross, Tare and Net display with stability indicator; one-tap tare and zero.

QR / barcode (Code 128, Code 39, EAN-13, EAN-8) with prefix and placeholder data.

Automatic serial-number generation (prefix-aware, e.g. GS-00012).

Unit selector (g / mg) and per-template gap / density / speed control.

Local SQLite storage; reports filterable by date range and product, exportable to CSV / XLSX.

Built on open hardware (ESP32) and open protocols (BLE GATT + TSPL).

# 2. System Overview

The solution is built around three nodes that talk over two well-defined links. The weighing scale streams ASCII weight data to the ESP32 over RS232 (level-shifted via MAX3232). The ESP32 hosts a BLE GATT service that the mobile app subscribes to. When the operator taps PRINT in the app, the active label template is serialised to a compact JSON envelope, written to the BLE command characteristic, and the ESP32 translates each element into TSPL commands sent to the TSC TTP-244 Pro over UART.

## Block diagram

+--------------+   RS232   +-------------+   BLE GATT   +------------+
  | Weighing     |---------->| ESP32       |<------------>| Mobile App |
  | Scale (ASCII)|           | Bridge      |              | (Flutter)  |
  +--------------+           +------+------+              +------+-----+
                                    | TSPL/UART                  |
                                    v                            v
                          +-------------------+        +-------------------+
                          | TSC TTP-244 Pro   |        | Local SQLite      |
                          | Thermal Printer   |        | Reports CSV/XLSX  |
                          +-------------------+        +-------------------+

# 3. Hardware

## 3.1 Bill of materials

| # | Item | Spec | Qty | Notes |
| --- | --- | --- | --- | --- |
| 1 | ESP32 DevKit | WROOM-32 / 4 MB flash | 1 | Any variant with BLE |
| 2 | MAX3232 module | DB9 + breakout | 1 | Level-shift scale RS232 |
| 3 | DC-DC buck | 12 V → 5 V, 2 A | 1 | Powers the bridge |
| 4 | Enclosure | ABS, panel-mount | 1 | With cable glands |
| 5 | TSC TTP-244 Pro | 203 dpi thermal | 1 | Customer printer |
| 6 | Weighing scale | Continuous ASCII RS232 | 1 | Essae / Avery / Phoenix Alpha etc. |
| 7 | USB-C cable | 0.5 m | 1 | For ESP32 power / debug |
| 8 | Jumper wires | Female-female | 8 |  |

## 3.2 Pin map

| Signal | ESP32 pin | Connects to | Notes |
| --- | --- | --- | --- |
| Scale RX | GPIO16 (RX1) | MAX3232 RO → Scale RS232 TX | Never connect RS232 voltages directly |
| Scale GND | GND | Scale ground | Common ground required |
| Printer RX | GPIO17 (TX2) | TSC DB-9 pin 3 | Use level shifter if printer expects ±12 V |
| Printer GND | GND | TSC DB-9 pin 5 |  |
| Status LED | GPIO2 | (built-in) | Lit when BLE client connected |
| Power | 5 V / VIN | Buck output | Or USB-C for development |

## 3.3 Notes on the TSC TTP-244 Pro

Speaks TSPL/TSPL2 (its native language). No driver required — plain text over the serial port.

Default serial: 9600 8N1. Some units ship at 19200; check the label on the printer.

Print head is 203 dpi → 8 dots per millimetre. The designer uses this conversion.

Gap mode is appropriate for die-cut labels; use BLINE for black-mark stock and adjust DIRECTION.

# 4. ESP32 Firmware

The firmware is a single Arduino sketch (GoldSilverLabelPrinter.ino) plus the ArduinoJson library. It uses both UART1 (for the scale) and UART2 (for the printer), and exposes a custom BLE GATT service.

## 4.1 BLE GATT layout

| UUID | Characteristic | Direction | Payload |
| --- | --- | --- | --- |
| 6f0a0001-… | Service | — | Container |
| 6f0a0002-… | Weight | NOTIFY | {"g":..,"t":..,"n":..,"s":0¦1,"ts":..} |
| 6f0a0003-… | Command | WRITE | JSON command envelope (see below) |
| 6f0a0004-… | Status | NOTIFY | {"status":"ok¦err","msg":".."} |

## 4.2 Command protocol

Print a label (the elements list is built by the app's designer):

{
  "cmd": "print",
  "label": {"w": 50, "h": 25, "gap": 3},
  "copies": 1,
  "elements": [
    {"type":"text","x":10,"y":10,"font":"3","text":"GOLD 22K"},
    {"type":"text","x":10,"y":40,"text":"Net: 12.345 g"},
    {"type":"qr",  "x":250,"y":10,"size":4,"data":"GS-0001|22K|12.345"},
    {"type":"bar", "x":10, "y":120,"btype":"128","data":"GS0001","height":60}
  ]
}

Other commands:

{"cmd":"tare"}   — capture current weight as tare

{"cmd":"zero"}   — clear tare offset

{"cmd":"feed","mm":5}   — feed paper

{"cmd":"status"} — return current snapshot

{"cmd":"raw","tspl":"..."} — escape hatch for arbitrary TSPL

## 4.3 Scale parser

The continuous ASCII parser collects bytes until \r or \n, looks for common stability flags ("ST", "S ") and net/gross markers ("NT", "NET"), then extracts the first signed floating-point number. The parser tolerates the leading mode bytes and varying whitespace produced by Indian shop scales. Unit conversion (kg → g) is applied automatically.

Examples it handles:

ST,GS,  +12.345 g\r\n   ->  grams=12.345  stable=true  net=false
US,NT,  +00.000 g\r\n   ->  grams=0.000   stable=false net=true
  +12.345 g\r\n          ->  grams=12.345  stable=false net=false

## 4.4 TSPL builder

The firmware exposes small helpers (tsplBegin, tsplText, tsplBarcode, tsplQR, tsplBox, tsplLogo, tsplPrint) that emit the canonical TSPL command sequence. Coordinates passed in are in printer dots (8 dots = 1 mm). The builder always issues SIZE, GAP, DENSITY, SPEED, DIRECTION and CLS before drawing — this guarantees the printer is in a known state for each job and avoids the common "label kept blank" bug.

# 5. Mobile App (Flutter)

The app is a Flutter project targeting Android 7+ and iOS 13+. State is managed with the Provider package; BLE is handled by flutter_blue_plus; persistence is via sqflite. The app is structured into five screens accessible from a bottom navigation bar.

## 5.1 Screens

| Screen | Purpose |
| --- | --- |
| Scale | Live Gross/Tare/Net, product picker, copies, big PRINT button |
| Designer | Drag-and-drop canvas at true label dimensions; element inspector |
| Templates | List of saved templates with metadata and preview |
| Reports | Date-range / product filter, totals, CSV / XLSX export |
| Device | BLE pairing, connection status, troubleshooting |

## 5.2 Label designer

The designer renders a fixed-DPI canvas sized to the chosen label dimensions in mm. Elements are positioned in printer dots so the on-screen layout maps exactly to what the printer produces. Each element has an inspector that exposes only the properties relevant to its type (font/scale/rotation for text; ECC level/size for QR; type/height for barcode; etc.). Elements can carry placeholders that the print-time binder substitutes with live values.

## 5.3 Placeholder reference

| Placeholder | Resolved at print-time to |
| --- | --- |
| {net} | Latest stable net weight with selected unit |
| {gross} | Latest gross weight |
| {tare} | Current tare offset |
| {serial} | Next serial number (auto-incremented) |
| {date} | Current date (dd-MM-yyyy) |
| {time} | Current time (HH:mm) |
| {product} | Product / item name entered on Scale screen |
| {purity} | Purity entered on Scale screen |
| {rate} | Rate per gram entered on Scale screen |
| {amount} | rate × net, formatted with 2 decimals |

## 5.4 Data model

templates(id, name, width_mm, height_mm, gap_mm, json, updated)
products (id, code, name, purity, hsn, rate, making, template_id)
prints   (id, serial, product, purity, gross_g, tare_g, net_g,
          rate, amount, ts, template)

# 6. End-to-end workflows

## 6.1 First-time setup

Wire the ESP32 to the scale (UART1) and the printer (UART2). Power on.

On the phone, open the app and tap the Device tab → Scan & connect.

Once connected, switch to Designer and create a template (e.g. 50 × 25 mm).

Drop Text, Weight, Serial, QR / Barcode elements; configure prefixes and fonts.

Save the template. It will appear in the Scale screen's template dropdown.

## 6.2 Routine printing

Place item on the scale.

Optionally tap TARE if a container is on the pan.

Verify Net weight, choose product / purity / rate / copies.

Tap PRINT LABEL. The label prints and the row is logged in the SQLite prints table.

## 6.3 Daily / monthly reports

Open the Reports tab.

Choose date range, optionally filter by product.

Header bar shows total net weight and total billed amount.

Tap CSV or XLSX to share via WhatsApp, email or save to drive.

# 7. Non-functional considerations

## 7.1 Security

BLE link is point-to-point; bond/pair the bridge once and trust thereafter.

Add a custom 4-digit unlock command (e.g. {"cmd":"unlock","pin":"1234"}) before accepting print jobs in v1.1.

All data is stored locally; nothing leaves the phone unless the operator exports a report.

## 7.2 Performance

Weight notifications throttled to 5 Hz — smooth UI without flooding the BLE link.

Print job round-trip is typically 200–400 ms for a 50×25 mm label.

BLE MTU is requested at 247 bytes so most jobs fit in one write.

## 7.3 Known limitations & roadmap

Logo printing currently uses PUTBMP against a file pre-loaded in printer flash. v1.1 will add chunked logo upload from the phone (DOWNLOAD BMP).

Tare offset is held in RAM. v1.1 will persist it via the ESP32 Preferences library.

No GST / HSN compliance fields wired into the bill yet. The products table already has the columns; add a UI in v1.1.

Optional cloud sync (Firebase) for multi-counter / multi-shop deployments is a planned v2 feature.

# 8. Deliverables in this drop

| Path | Contents |
| --- | --- |
| firmware/GoldSilverLabelPrinter.ino | ESP32 firmware (BLE + scale + TSPL) |
| firmware/README_FIRMWARE.md | Wiring, libraries, BLE GATT, test recipe |
| app/pubspec.yaml | Flutter dependencies |
| app/lib/main.dart | App entry point & theme |
| app/lib/services/ble_service.dart | BLE scan/connect, weight stream, command writer |
| app/lib/services/db_service.dart | SQLite store for templates / products / prints |
| app/lib/models/label_element.dart | Element model + placeholder substitution |
| app/lib/pages/home_shell.dart | Bottom-nav shell |
| app/lib/pages/scale_page.dart | Live readings + PRINT button |
| app/lib/pages/designer_page.dart | Drag-and-drop canvas designer |
| app/lib/pages/templates_page.dart | Saved template list |
| app/lib/pages/reports_page.dart | Date-range reports, CSV/XLSX export |
| app/lib/pages/connect_page.dart | BLE pairing UI |
| app/README_APP.md | Build & run instructions |
| mockups/app_screens_mockup.html | Visual mockups of all 5 screens + block diagram |
| docs/Gold_Silver_Label_App_Architecture.docx | This document |

## Document end

Once the firmware and app are validated against your scale and printer, we can add the GST/HSN billing layer, the logo upload pipeline, and the multi-shop cloud sync as a v1.1 / v2 increment.